SHIPPER’S COMPANY  LETTERHEAD

BOOKING NR.	: 							      DATE :

CONTAINER NR.: 					VEHICLE  IDENTIFICATION:

SHIPPER’S DECLARATION

FOR IMDG SPECIAL PROVISION 962 COMPLIANT SHIPMENT

(*check (√) what is valid):

| UN | PROPER SHIPPING NAME | * |
| --- | --- | --- |
| 3166 | VEHICLE, FLAMMABLE GAS POWERED |  |
| 3166 | VEHICLE, FLAMMABLE LIQUID POWERED |  |
| 3166 | VEHICLE, FUEL CELL, FLAMMABLE GAS POWERED |  |
| 3166 | VEHICLE, FUEL CELL, FLAMMABLE LIQUID POWERED |  |
| 3171 | BATTERY-POWERED VEHICLE |  |
| 3171 | BATTERY-POWERED EQUIPMENT |  |

The undersigned, hereby declares that the vehicles and/or equipment offered for shipment fully comply with the relevant requirements of IMDG code (Amendment 41-22) Special Provisions 962:

“Vehicles, not meeting the conditions of special provision 961 shall be assigned to class 9 and shall meet the following requirements:

.1	vehicles shall not show signs of leakage from batteries, engines, fuel cells, compressed gas cylinders or accumulators, or fuel tank(s) when applicable;

.2	for flammable liquid powered vehicles the fuel tank(s) containing the flammable liquid shall not be more than one fourth full and in any case the flammable liquid shall not exceed 250 L unless otherwise approved by the competent authority;

.3	for flammable gas powered vehicles, the fuel shut-off valve of the fuel tank(s) shall be securely closed;

.4	installed batteries shall be protected from damage, short circuit, and accidental activation during transport. Lithium batteries shall meet the provisions of 2.9.4, except that 2.9.4.1 does not apply when pre-production prototype batteries or batteries of a small production run, consisting of not more than 100 batteries, are installed in the vehicle and the vehicle is manufactured and approved according to the provisions applied in the country of manufacture or country of use. Where a lithium battery installed in a vehicle is damaged or defective, the battery shall be removed and transported according to SP 376, unless otherwise approved by the competent Authority.

The provisions of this Code relevant to marking, labelling, placarding and marine pollutants shall not apply.”

PLACE AND DATE:

NAME AND SIGNATURE OF PACKING CO./PERSON: